|  |  | JC Fábrica de válvulas |  |  |  |  |  | PRESSURE TEST CERTIFICATE EN-10204 3.1 |  |  |  |  |  |  | PS07/F5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Product Group |  |  | TRUNNION BALL VALVE |  |  |  |  |  | Report No (RS-PSS) |  | RS-250117-PSS |  |  |  |  |
| Size |  |  | 10” |  | Class: | 300 |  |  | Route Sheet |  | WOJCB243405 |  |  |  |  |
| Valve |  |  | 6030AICG250LPT |  |  |  |  |  | Date of report |  | 17/01/2025 |  |  |  |  |
| Valve SN. |  |  | 1- 248966-3 |  |  |  |  |  | Customer |  | JC/TL BALL VALVE CORPORATION |  |  |  |  |
| Body Material |  |  | ASTM A350 LF2 CL.1/A105N |  |  |  |  |  | Customer PO |  | JCTL-9336 |  |  |  |  |
| Quantity |  |  | 3 |  |  |  |  |  | Item No |  | 3 |  |  |  |  |
| End / Bore |  |  | RF / FB |  |  |  |  |  | Testing Standard |  | EN 12266-1 / API 598 |  |  |  |  |
| INSTRUMENTS: |  |  |  |  |  |  |  |  |  |  |  |  | LOCATION: SPAIN |  |  |
| PRESSURE GAUGE NUMBER |  |  |  |  |  |  | DUE DATE |  |  |  | MEDIUM |  |  |  |  |
| 0250-001 (0-250 bar) |  |  |  |  |  |  | 09.04.2025 |  |  |  | DM WATER |  |  |  |  |
| 0016-005 (0-16 bar) |  |  |  |  |  |  | 09.04.2025 |  |  |  | AIR |  |  |  |  |
| TEST PARAMETERS AND TEST RESULTS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PRESSURE RATING (barg) |  |  |  |  |  |  |  |  |  | TESTING DURATION (seconds) |  |  |  |  |  |
| SHELL HYDRO |  |  |  | SEAT HYDRO |  |  | SEAT AIR |  |  | SHELL HYDRO | SEAT HYDRO | SEAT AIR |  | RESULT |  |
| 78 |  |  |  | 56 |  |  | 6 |  |  | 120 | 120 | 120 |  | OK |  |
| SL.NO | VALVE SL. NO |  |  |  |  |  | FUNCTIONAL TEST |  |  | SHELL HYDRO | SEAT HYDRO | SEAT AIR |  | RESULT |  |
| 1 | 1-248966-3 |  |  |  |  |  | Satisfactory |  |  | OK | OK | OK |  | OK |  |
| EVALUATION OF RESULTS: DECISION REJECTED ACCEPTED INSPECTOR: Ethan Martos |  |  |  |  |  |  |  |  |  |  | OBSERVATIONS: |  |  |  |  |